Rotary Evaporator EV400H

Technical Specifications:

- Lifting Mechanism: Manual lift, lifting stroke: 160 mm

- Micro switch controls locking and release of the lift; can be operated easily with one hand

- ★ Rotation Speed Range: 20–300 rpm, LCD digital display

- Condenser: Diagonal serpentine condenser, 1200 cm² standard condensation area; optional 1500 cm² condenser available

- Adjustable angle of rotary joint: 0–60°

- Evaporating Flask Capacity: 50 mL – 3000 mL

- ★ Heating Bath Temperature Range: Room temperature to 210 °C, switchable between water/oil bath

  - Bath capacity: 5 L

  - Bath material: 304 stainless steel; optional PTFE coating

- Temperature Control: Intelligent PID control (LCD display)

  - Temperature accuracy: ±1 °C (water), ±2 °C (oil)

- Dry-boil protection with automatic power-off

- Over-temperature alarm for water bath

- Timer function for heating bath

- ★ Single-button shuttle control knob; supports forward/reverse rotation

- Optional digital vacuum controller for precise vacuum regulation during distillation

Standard Configuration:

- Rotary evaporator main unit ×1

- Heating oil bath ×1

- Diagonal double-layer serpentine condenser ×1

- Evaporating flask ×1 (1000 mL)

- Receiving flask ×1 (1000 mL)