Rotary Evaporator EV400 VAC

Technical Specifications:

- ▲ Lifting Mechanism: Motorized automatic lift; lifting stroke: 160 mm

- ▲ Rotation Speed Range: 20–300 rpm

- Condenser: Vertical serpentine condenser, 1200 cm² condensation area

- Adjustable angle of rotary joint: 0–60°

- Automatic lift upon power failure to separate evaporating flask from heating bath, preventing localized overheating

- ▲ Secondary condensation recovery system ensures high sample recovery and protects vacuum pump from solvent corrosion

- Evaporating Flask Capacity: 50 mL – 3000 mL

- ▲ Heating Bath Temperature Range: Room temperature to 210 °C, switchable between water/oil bath

  - Bath capacity: 5 L

  - Bath material: 304 stainless steel

- Temperature Control: Intelligent PID control (LCD display)

  - Temperature accuracy: ±1 °C (water), ±2 °C (oil)

- Dry-boil protection with automatic power-off

- Over-temperature alarm for water bath

- Built-in digital vacuum controller

- ▲ 4.3-inch color touchscreen controller with simultaneous digital display of vacuum, rotation speed, and other parameters; includes graphical distillation curve

- ▲ Gradient distillation mode enables automatic program control of vacuum and rotation speed; built-in intake valve automatically releases vacuum and stops distillation at program end

- Built-in solvent database with over 40 solvents for one-touch distillation

- Optional digital vapor temperature display for precise monitoring

- ▲ Optional automatic distillation function automatically identifies solvent boiling point and optimizes distillation conditions—no manual tuning required

- Intelligent timed distillation function

- USB data interface for easy export of experimental data

- Intelligent vacuum system allows vacuum controller to manage pump operation—reducing lab noise and vibration, and extending pump lifespan

Standard Configuration:

- Rotary evaporator main unit ×1 (includes built-in digital vacuum controller and color touchscreen controller)

- Heating oil bath ×1

- Vertical serpentine condenser ×1

- Gas-liquid separation flask ×1

- Evaporating flask ×1 (1000 mL)

- Receiving flask ×1 (1000 mL)

Click the card above to download the translated Word document. Let me know if you'd like all three EV400 models summarized in a comparison table or formatted for a product brochure.