Air Compressor Technical Parameters

Model: Air 204 Brand: LabTech Manufacturer: LabTech

1. Purpose:

The oil-free air compressor provides a continuous supply of pure, dry compressed air with stable and reliable performance. It is used to support instruments such as Atomic Absorption Spectrophotometers, ICP, Nitrogen Evaporators, etc.

2. Technical Parameters:

- Pressure regulation range: 0.005–0.5 MPa

- Air flow rate: ≤ 20 L/min

- Automatic drainage, utilizing a spiral centrifugal air-cooled water separator that cools the compressed air while centrifugally separating condensate, achieving rapid and high efficiency

- ★ Electronic dehumidification function, suitable for humid southern regions, provides more thorough water removal

- Utilizes a twin-cylinder piston compressor, ensuring stable operation, requires no lubrication, and delivers pure, stable, oil-free output gas

- Efficient two-stage filtration system: includes intake filtration and exhaust filtration for purer gas

- Enclosed cabinet design with wavy sound-absorbing cotton for effective noise reduction; overall machine noise < 60 dB

- Manufacturer is certified under the ISO9001 production management system