Refrigeration Heating Circulator

Model: RH25-12A
Brand: LabTech
Manufacturer: LabTech

Technical Parameters:

- ★ Temperature Control Range: -25°C to 100°C; Control Accuracy: ±0.2°C

- Control Method: Intelligent PID control (LCD digital display), combining PID digital temperature control technology with Hot Gas Bypass technology.

- Cooling Method: Compressor refrigeration; Heating Method: Electric heating

- ★ Heating Power: 2000W; Refrigeration Power: 650W @20°C; Eco-friendly Refrigerant: R134A

- ★ Pump Flow Rate: 5 L/min; Pump Pressure: 0.2 Bar

- ★ Tank Volume: 12L; Bath Opening Dimensions: ≥240mm × 140mm

- Dual internal/external circulation switching, functions as both a high-precision water bath and a circulating water chiller.

- High/low temperature and water level alarm functions.